STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v3.51 NDIS v3.1 Driver

For EtherPower, EtherPower^2, and the EtherPower 10/100 Family

of PCI Network Cards

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1996 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Supported network cards

o  An overview

o  Preinstallation hardware and software requirements

o  Supplied files

o  Network card installation notes

o  Dual channel support

o  Driver installation

- Primary installation (network card not presently installed)

- Network card already installed or older driver previously loaded

o  Modifying the data rate

SUPPORTED NETWORK CARDS

o  EtherPower Ethernet network cards

(SMC8432 board types)

o  EtherPower^2 Dual-Channel Ethernet network cards

(SMC8434 board types)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332DST board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-TX network cards

(SMC9332BDT board type)

o  EtherPower 10/100 Dual-Channel

Fast Ethernet 100BASE-TX network cards

(SMC9334BDT board type)

o  EtherPower 10/100 Fast Ethernet 100BASE-T4 network cards

(SMC9332BVT board type)

OVERVIEW

This file explains how to install the Windows NT driver for x86-based

platforms.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with 80386 or higher

microprocessor and PCI bus.

- An EtherPower, EtherPower^2, or EtherPower 10/100 PCI

network card for that bus.

Software Requirements:

- Windows NT v3.51

SUPPLIED FILES

General files

o  NT_351.DOC       This file

o  RELEASE.DOC      Release notes for this version of the driver

Files to install the driver on Windows NT

o  SMCPWRN.SY_      The driver

o  SMCPWRN.HL_      The driver's associated help file

o  OEMSETUP.INF     The driver's setup file

NETWORK CARD INSTALLATION NOTES

Refer to the user guide for complete installation instructions.

When installing an SMC PCI network card in a PCI computer, make sure

the slot you select supports bus master operations.

o  Motherboards with bus master jumpers and interrupt jumpers for

PCI slots:

1. Enable bus master operations to the selected PCI slot using the

appropriate motherboard jumper.

2. Select interrupt to the selected PCI slot using the appropriate

motherboard jumper.

3. Enter your computer's setup menu and:

- Enable bus master on the PCI slot.

- Enable I/O on the PCI slot.

- Define interrupt number on the PCI slot.

- Modify CMOS settings to make an IRQ available to the PCI bus.

o Motherboards with automatic PCI parameterization:

1. No specific setup is needed.

SMC9334BDT EtherPower 10/100 network card ONLY:

During the network adapter selection process, the SMC9334BDT

EtherPower 10/100 dual channel network card may be represented as two

SMC9332BDT EtherPower 10/100 adapters (one for each channel). If no

option for 'SMC9334BDT EtherPower 10/100 Dual Channel' is presented,

select the option for 'SMC9332BDT EtherPower 10/100' for both channels

SMC9332DST EtherPower 10/100 network card ONLY:

Ensure that the data rate (10 Mbps vs. 100 Mbps) configured for the

network card matches the speed of the hub to which it is attached.

The SMC9332DST EtherPower 10/100 network card is default configured

to autodetect the data rate. Refer to the MODIFYING THE DATA RATE

section later in this document.

SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower 10/100 network cards ONLY:

The Auto-Negotiation feature of the SMC9332BDT/SMC9334BDT/SMC9332BVT

EtherPower 10/100 network card will set the operating speed of the

network card (10 Mbps [MB10] or 100 Mbps [MB100]). If the attached hub

is full duplex capable and uses Auto-Negotiation, the SMC9332BDT

EtherPower 10/100 network card will configure itself for full duplex

operation. If the attached hub is full duplex capable, but does not

use Auto-Negotiation, you must manually specify full duplex mode.

Refer to the MODIFYING THE DATA RATE section later in this document.

DUAL CHANNEL SUPPORT

EtherPower^2 and SMC9334BDT EtherPower 10/100

The EtherPower^2 Dual Channel PCI Ethernet and the SMC9334

EtherPower 10/100 Fast Ethernet network cards conform to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO

UNIX. Other networking environments may not support the unique

dual channel capability of the EtherPower^2 or the SMC9334

EtherPower 10/100 network card.

DRIVER INSTALLATION

o Primary installation (network card not presently installed)

If you've already installed another card or loaded another driver, refer

to the procedure titled 'Network card already installed or older driver

previously loaded'.

1.  Complete the Windows NT installation without the SMC network card

installed in your PC. Have the SMC SuperDisk and Windows NT

installation files (on CD or hard drive) available.

2.  Start Windows NT and double-click the 'Control Panel' and 'Network'

icons.

3.  You will be prompted to install networking functionality. Click 'Yes'.

4.  Follow the screen prompts. When prompted, insert the Windows NT CD or

specify the path to those installation files on your hard drive.

5.  When prompted with the 'Network Adapter Card Detection' dialog box,

select 'Do Not Detect'.

6.  When prompted with the 'Add Network Adapter' window, scroll to and

select '<Other> Requires disk from manufacturer'.

7.  Select 'Continue'.

8.  When prompted, insert the SuperDisk, specify the path to the root

directory (e.g., A:\), and click 'OK'.

9.  Select 'SMC EtherPower Family Network Card' and click 'OK'.

10. When prompted, specify the network card data rate. 'AutoSense' is

the recommended setting for all EtherPower family network cards.

Click 'Continue'.

11. At this point, you can install additional protocols and network

components via the 'Windows NT Setup' window. Click 'Coninue'

when finished.

12. Verify the network card type and settings. When presented with the

'Network Settings' window, click on 'OK'.

NOTE:  If you select the IPX/SPX protocol, the 'NWLINK IPX/SPX

Protocol Configuration' window appears, permitting customization

of the protocol. Click 'OK'.

13. Follow the screen prompts as directed. Enter the 'Domain' or

'Workgroup' names when prompted (optional) and click 'OK'.

14. When prompted. select 'Restart Now'.

NOTE:  In the unlikely event you encounter any device driver errors after

Windows NT boots, check the Event Log (under 'Administrative Tools'

icon) to see whether the error is related to your SMC network card.

o Network card already installed or older driver previously loaded

Follow this procedure to replace Windows NT driver files with the latest

SMC driver files. By removing driver files, you will lose network settings.

Make sure you have a record of settings such as IP address, workgroup

name, etc. Check with your network administrator for complete information.

1.  Install the SMC network card, attach network cable, and boot Windows NT.

2.  Double-click the 'Control Panel' and 'Network' icons.

3.  When prompted with the 'Network Settings' window, highlight the network

card you wish to remove, click on 'Remove', and click on 'OK'.

4.  Select 'Restart Now' when prompted.

5.  Start Windows NT and double-click the 'Control Panel' and 'Network'

icons. The 'Network Settings' window appears. Select 'Add Adapter'.

6.  When prompted with the 'Add Network Adapter' window, scroll to and

select '<Other> Requires disk from manufacturer'.

7.  Select 'Continue'.

8.  When prompted, insert the SuperDisk, specify the path to the root

directory (e.g., A:\), and click 'OK'.

9.  Select 'SMC EtherPower Family Network Card' and click 'OK'.

10. When prompted, specify the network card data rate. 'AutoSense' is

the recommended setting for all EtherPower family network cards.

Click 'Continue'.

11. At this point, you can install additional protocols and network

components via the 'Windows NT Setup' window. Click 'Coninue'

when finished.

12. When prompted, select 'Restart Now'.

NOTE:  In the unlikely event you encounter any device driver errors after

Windows NT boots, check the Event Log (under 'Administrative Tools'

icon) to see whether the error is related to your SMC network card.

MODIFYING THE DATA RATE

1. Double-click on the 'Control Panel' icon.

2. Double-click on the 'Network' icon.

3. Highlight the SMC Adapter in the 'Installed Adapter Cards' list.

4. Click on the 'Configure' button.

5. Scroll to the data rate setting you wish to use and click on 'OK'.

'AutoSense' is the recommended setting for all EtherPower Family

network cards.

6. Click the 'Close' button and restart your computer.

SMC9332DST EtherPower 10/100 ONLY:

The default parameter for DATA RATE is autodetect. This default

setting will automatically detect the line speed at power up only.

If autodetect failed to work, ensure that the data rate (10 Mbps

or 100 Mbps) configured for the network card matches the speed of

the hub to which it is attached.

SMC9332BDT/SMC9334BDT/SMC9332BVT EtherPower 10/100 ONLY:

This network card supports NWAY (Auto-Negotiation). Setting the

DATA RATE to AUTOSENSE will allow the driver to automatically

detect the line speed (10 Mbps or 100 Mbps) and operate at the

highest possible data rate.

The recommended setting for the DATA RATE is AUTOSENSE. If you

manually set the DATA RATE to anything other than AUTOSENSE,

(for example, either half-duplex or full-duplex operation), the

settings you select must be supported by the hub to which you

are attaching.

After completing the installation and configuration, you must reboot your

computer.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of Standard Microsystems Corporation. Other product and company names

are registered trademarks or trademarks of their respective holders.