SMC

SuperDisk

For EtherPower, EtherPower^2, and EtherPower 10/100 Family

PCI Network Adapters

This software is licensed by SMC for use by its customers only.

Copyright (c) 1997 SMC Networks, Inc.. All rights reserved.

Version 5.0         14 November 1997

SUPERDISK CONTENTS

The following list describes some of the files contained on this

SuperDisk:

File             Contents

README.DOC       This file.

Utilities/       Contents

Drivers

DOS ODI          Open data-link interface driver to be used with

ODI protocol stacks.

EZSTART          Documentation for the EZStart Program.

EZDOSODI         Batch file that performs an EZStart macro for

automatic NetWare client installation.

NDIS             DOS NDIS drivers to be used with NDIS-

compliant network operating systems and protocol

stacks. Contain documentation with supporting

files information for specific network operating

systems.

NETWARE        Server driver for Novell NetWare 386 version 3.12.

386

NETWARE          Driver for Novell NetWare 386 v4.x.

386 4.x

WINDOWS for      NDIS 3 Driver for Microsoft Windows for

WORKGROUPS       Workgroups version 3.11.

WINDOWS NT       Drivers for Microsoft Windows NT v3.51 & v4.0.

WINDOWS 95       Driver for Microsoft Windows 95.

In addition to the driver or utility program itself, each

driver has the following associated files:

o  An INSTALL.DOC or other .DOC file that explains how to install

and use the software

o  A RELEASE.DOC file that states the current version, release date,

and any improvements made in this version of the software

SUPPORTED ADAPTERS

o  EtherPower Ethernet PCI adapters (8432 board types)

o  EtherPower^2 Dual-Channel PCI Ethernet adapters (8434 board types)

o  EtherPower 10/100 Fast Ethernet PCI adapters (9332 board types)

DISK INFORMATION

This SMC SuperDisk is designed to be used with SMC's EZStart Program,

which is on this SuperDisk.  EZStart configures and tests your

adapter, copies the files you need from this disk to your PC, and

displays or prints appropriate instruction files that are located on

this disk.

SYSTEM REQUIREMENTS

To use EZStart, your system must meet the following minimum

requirements:

o  MS-DOS or PC-DOS operating system, v3.11 or later

o  To take advantage of EZStart's graphics features, a VGA or better

monitor; however, EZStart can operate in text mode on any monitor

o  Approximately 380 K run-time memory to run EZStart.

o  A minimum amount of available hard disk space (typically 50 KB)

for installing a single driver from the SuperDisk using EZStart

OR

At least 822 KB of available hard disk space if you intend to

extract all of the drivers from the SuperDisk

Note: Refer to the "Installing A Network Driver" section later in

this document or the user guide for more information on

copying driver files from the SuperDisk.

USING EZStart

EZStart is an all-purpose utility program that simplifies the

following tasks:

o Automatically configure and test the adapter

o Install the network driver

In addition, EZStart makes it easy to perform optional installation

and diagnostic procedures:

o Manually configure the adapter by selecting a value for

each configuration option on an individual basis

o Test two or more adapters communicating with each other

over your actual network

o View the configurations of all installed adapters

LOADING EZStart

EZStart can run from the diskette, or for faster operation it can be

copied onto a hard disk and run from there.

Note: To run EZStart from the hard disk, copy all the files from

the SuperDisk to your computer's hard disk.  However, make sure

that the compressed driver file, SMCHnn.EXE, and EZSTART.EXE

reside in the same directory.

o To load EZStart from the DOS command line, type:

[path]ezstart<Enter>

o To load EZStart in Text Mode (instead of Graphics Mode), type

[path]ezstart/T<Enter>

o To load EZStart from Microsoft Windows, select the File menu in

Program Manager.  When the Run dialog box appears, type:

[path]ezstart

in the Command Line box, then select the OK button.

INSTALLING A NETWORK DRIVER

EZStart accesses network driver files through the compressed

driver file, SMCHnn.EXE.  The file name, SMCHnn.EXE, specifies

the family of adapters supported and the SuperDisk version. In

the file name SMCH50.EXE, for example, "H" denotes the newer

EtherPower PCI Adapter and "50" denotes SuperDisk v5.0.

EZStart copies the network driver and any other supporting files

from the compressed driver file to a location compatible with

your network operating system.  If your network operating system

requires an installation or options floppy diskette, or your PC

has a single floppy drive, you can build the installation or

options diskette on the SuperDisk by following these steps:

o Make a backup copy of your original SuperDisk

o Load EZStart

o When prompted for the Destination Directory by the EZStart

Driver Installation window, specify the floppy drive which

contains the SuperDisk (e.g., a:\)

Note: If you prefer, you can extract all of the network drivers

from the SuperDisk by:

o Changing your default directory to the floppy drive where

the SuperDisk resides and,

o Typing SMCH50 <Destination Directory> at the DOS prompt

(where <Destination Directory> is the path where you want

the extracted SuperDisk contents to reside)

Note: There must be at least 822 KB of available hard disk space

TECHNICAL SUPPORT

For technical support, first contact your place of purchase. If you

need additional assistance, call our Technical Support departments:

From U.S.A. and Canada (8:30 am - 8:00 pm Eastern Time)

(800) SMC-4-YOU

(516) 435-6250

(516) 434-9314 (Fax)

From Europe (8:00 am - 5:30 pm UK Greenwich Mean Time)

44 (0) 1344 420068

44 (0) 1344 418835 (Fax)

SMC offers Bulletin Board Services (BBS) from which you can obtain

the latest information on adapters and drivers for SMC products via

modem:

New York      (516) 434-3162

up to 28,800 baud, 8 data bits, no parity, 1 stop bit

Germany       49 (89) 928806-50

up to 28,800 baud, 8 data bits, no parity, 1 stop bit

France        33 (1) 39.73.57.00

up to 14,400 baud, 8 data bits, no parity, 1 stop bit

United        44 (0) 1344 418838

Kingdom      up to 28,800 baud, 8 data bits, no parity, 1 stop bit

INTERNET Support:  SMC Technical Support can also be obtained through

the INTERNET. Our address is:  techsupport@smc.com or

european.techsupport@smc.com.

CompuServe Support:  For information and drivers for SMC network

products, you can also access the SMC Forum through CompuServe. Log

into CompuServe, and at the "!" prompt type "GO SMC".

World Wide Web:  http://www.smc.com

FTP Server:  ftp.smc.com

Login as anonymous using email address as password. Binary files

must be transferred using the binary keyword or option.

EliteFax (SMC's Fax-on-Demand System):

U.S.A. and Canada:  (800) SMC-8329

Elsewhere:  (516) 435-6107

TROUBLESHOOTING TIPS

o PCI LOCAL BUS SPECIFICATION v2.0 COMPATIBILITY

Because PCI is a relatively new industry specification, there are a

number of compatibility issues which may affect the successful

installation of any PCI adapter in your computer. SMC urges you to

contact the manufacturer of your computer to determine if your BIOS

correctly supports the features of the PCI Local Bus Specification

(v2.0).

Many early versions of PCI BIOS do not properly support the PCI

specification. If you have an early PCI BIOS version, your computer

may hang when a LAN driver attempts to load. This problem can

normally be resolved with a BIOS upgrade from your computer

motherboard manufacturer.

PCI Local Bus Compatibility Warning Message

(ODI Driver Only)

When loading an ODI driver, you may receive the following

warning message:

"This machine host to PCI bridge has a known bug.

Driver limits DC21040 bursts as a workaround."

This message indicates that your computer does not correctly support

the PCI Local Bus Specification (v2.0). The SMC driver will work

in this environment, but you are urged to contact the manufacturer of

your computer to determine if your BIOS correctly supports the

features of the PCI Local Bus Specification (v2.0).

o SUPPORTED PLATFORMS

SMC has developed a comprehensive list of compatible PCI platforms

to assist you in determining whether your computer properly supports

the PCI specification. This constantly updated list is available

via EliteFax, SMC's fax-on-demand system. To have this list faxed

directly to you, call (touch-tone) 800.SMC.8329 (U.S.A. and Canada)

or 516.435.6107 (elsewhere).

o DUAL CHANNEL SUPPORT (EtherPower^2 Only)

The EtherPower^2 Dual Channel PCI Ethernet adapter conforms to the

PCI-to-PCI Bridge Architecture Specification, Revision 1.0, April 5,

1994. Networking environments that support dual channel operation

include Novell server (32-bit ODI driver), Windows NT, and SCO. Other

networking environmnets do not support the unique dual channel capability

of the EtherPower^2 adapter. These environments include Novell client

(DOS ODI driver), Windows for Workgroups v3.11, and networking

environments that use DOS or OS/2 NDIS2 drivers.

o PCI COMPUTERS WITHOUT SELF-CONFIGURING FEATURES

Some PCI computers are not self-configuring. In such computers, you

may have to perform some or all of the following functions by

motherboard jumper changes and/or BIOS commands:

-   The PCI slot must be configured to support bus-mastering.

-   The PCI slot must be configured to support INTA.

-   PCI INTA must be assigned to a free IRQ (interrupt) number.

Consult your computer manual for information on changing motherboard

jumper settings and BIOS SETUP parameters for use with PCI adapters.

o EMM386 MEMORY-MANAGEMENT UTILITY

EMM386 v4.48 or Earlier

If you are using EMM386 v4.48 or earlier (provided with DOS v6.21

and earlier), upgrade to EMM386 v4.49 by obtaining DOS v6.22 or later,

or use a different memory manager. Otherwise, incompatibilities in

EMM386 may cause your computer to hang when a LAN driver attempts to

load. To determine your DOS version, type VER and press Enter at the

DOS prompt.

Memmaker and EMM386

Using the DOS v6.21 or v6.22 Memmaker program may cause system

reboots. Selecting Memmaker's Aggressive memory scanning option will

cause Memmaker to specify HIGHSCAN as an option parameter to EMM386

in the CONFIG.SYS file. The HIGHSCAN option may cause EMM386 to map

memory into the F000:0 BIOS space used by the system BIOS. In this

instance, EMM386 does not recognize that the BIOS may be using the

entire 64 KB range from F000:0 to F000:FFFF. To resolve this problem,

do not select the Aggressive memory scan option on Memmaker.

o SPECIFYING THE PCI LOGICAL SLOT

The EtherPower^2 operates as two separate single-channel PCI adapters

occupying a single physical PCI slot. Each EtherPower^2 channel is

automatically assigned a logical slot number based on the number of PCI

devices installed in your computer.

For example, if the EtherPower^2 is the only PCI device installed in

your computer:

Channel 1 is assigned PCI slot number 16.

Channel 2 is assigned PCI slot number 17.

(By convention, slot numbers 0 through 15 are reserved for ISA or EISA

devices.)

If your computer already contains a single PCI device and you then install

the EtherPower^2 adapter:

The PCI device already installed is assigned PCI slot 16.

Channels 1 and 2 of the EtherPower^2 adapter are assigned PCI slot

numbers 17 and 18, respectively.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is SMC Networks, Inc.,  350 Kennedy,

Hauppauge,  NY    11788.

SMC is a registered trademark; and EtherPower,

EtherPower^2, EtherPower 10/100, EZStart, and SuperDisk are trademarks

of SMC Networks, Inc.. Other product and company names

are registered trademarks or trademarks of their respective holders.